9. PSK MODULATION AND DEMODULATION

9.1 Objective

To analyze a PSK modulation system and interpret the modulated and demodulated waveforms.

9.2 Hardware Required

PSK Trainer Kit

Dual Trace oscilloscope-POS-2020

9.3 Introduction

Phase shift keying is a modulation/data transmitting technique in which phase of the carrier signal is shifted between two distinct levels. In a simple PSK(ie., Binary PSK) un-shifted carrier Acost is transmitted to indicate a 1 condition, and the carrier shifted by 1800 ie.,  – Acost  is transmitted to indicate as 0 condition.

S(t)= Acos 2πfct            for Binary 1

Acos (2πfct  +π)    for Binary 0

WIRING /TRAINER KITDIAGRAM

Fig 9.1 Wiring Diagram for PSK Modulation and Demodulation

9.4 Pre Lab Questions

- What is BPSK?

- Draw the truth table, Phasor diagram, Constellation diagram of BPSK.

- What is bit error rate and give the expression for bit error Probability of BPSK.

- Compare binary PSK with QPSK.

- Write the applications of PSK.

9.5 Lab Procedure

- Connect the trainer to mains and switch on the power supply.

- Observe the output of the carrier generator using CRO,it should be an 17KHZ sine with 5Vpp amplitude.

- Observe the various data signals(4KHZ  and 8KHZ) using CRO

Modulation

- Connect carrier signal to carrier input of the PSK modulator.

- Connect data signal say 4KHZ from data source to data input of the modulator.

- Keep CRO in dual mode and connect CH1 input of the CRO to data signal and CH2 to the output of the PSK modulator.

- Observe the PSK output signal with respect to data signal and plot the waveforms.

Demodulation

- Connect the PSK output to the PSK input of the demodulator.

- Connect carrier to the carrier input of the PSK demodulator.

- Keep CRO in dual mode and connect CH1 to data signal (at modulator) and CH2 to the output of the demodulator.

- Compare the demodulated signal with the original signal. By this we can notice that there is no loss in modulation and demodulation process

- Repeat the steps 5 to 11 with different data signal ie., 8KHZ

9.6 Model Graph

Figure 9.2 PSK Waveforms for different data input signals

9.7 Observation-Hardware

PSK Modulation

| Signal Name | Amplitude | Time Period |
| --- | --- | --- |
| Carrier signal |  |  |
| Data source For 4KHz For 8KHz |  |  |
| Modulated output For 4KHz For 8KHz |  |  |

PSK Demodulation

| Signal Name | Amplitude | Time Period |
| --- | --- | --- |
| Demodulated output For 4KHz For 8KHz |  |  |

9.8 Post Lab Questions

1. Differentiate between coherent and non-coherent detection

2. What is the maximum Bandwidth of BPSK system?

3. What is the main advantage of M-ary PSK?

4. Represent the M-ary PSK signal mathematically.

5. Show the PSK modulated waveform for the message 10010110.

Lab Result

Thus the PSK modulation and demodulation were performed.